Workplace Assessment Checklist

|  | Working Conditions | YES | NO |
| --- | --- | --- | --- |
|  | The workstation should be designed or arranged so it allows the employee’s… |  |  |
| A | Head and neck to be about upright (not bent down/back) |  |  |
| B | Head, neck and trunk to face forward (not twisted) |  |  |
| C | Trunk to be about perpendicular to floor (not leaning forward/backward) |  |  |
| D | Shoulders and upper arms to be about perpendicular to floor (not stretched forward) and relaxed (not elevated) |  |  |
| E | Upper arms and elbows to be close to body (not extended outward) |  |  |
| F | Forearms, wrists and hands to be straight and parallel to floor (not pointing up/down) |  |  |
| G | Wrists and hands to be straight (not bent up/down or sideways toward little finger) |  |  |
| H | Thighs to be about parallel to floor and lower legs to be about perpendicular to floor |  |  |
| I | Feet to rest flat on floor or be supported by a stable footrest |  |  |
| J | VDU tasks to be organised in a way that allows the employee to vary VDU tasks with other work activities, or to take micro-pauses while at workstation |  |  |

|  | Seating The Chair | YES | NO |
| --- | --- | --- | --- |
| 1 | Backrest provides support for employee’s lower back (lumbar area) |  |  |
| 2 | Seat width and depth accommodate specific employee (seat pan not too big/small) |  |  |
| 3 | Seat front does not press against the back of the employee’s knees and lower legs (seat pan not too long) |  |  |
| 4 | Seat has cushioning and is rounded/has “waterfall” front (no sharp edge) |  |  |
| 5 | Armrests support both forearms while employee performs VDU tasks and do not interfere with movement |  |  |

|  | Keyboard/Mouse The keyboard/input device is designed or arranged for doing VDU tasks so that… | YES | NO |
| --- | --- | --- | --- |
| 6 | Keyboard/input device platform(s) is stable and large enough to hold keyboard and input device |  |  |
| 7 | Input device (mouse or trackball) is located right next to keyboard so it can be operated with reaching |  |  |
| 8 | Mouse is easy to activate and shape/size fits hand of specific employee (not too big/small) |  |  |
| 9 | Wrists and hands do not rest on sharp or hard edge |  |  |
|  | Monitor | YES | NO |
| 10 | Top line of screen is at or below eye level so employee is able to read it without bending head or neck down/back (For employees with bifocals/trifocals, see next item) |  |  |
| 11 | Employee with bifocals/trifocals is able to read screen without bending head or neck backward |  |  |

|  | Monitor | YES | NO |
| --- | --- | --- | --- |
| 11 | Employee with bifocals/trifocals is able to read screen without bending head or neck backward |  |  |
| 12 | Monitor distance allows employee to read screen without leaning head, neck or trunk forward/backward |  |  |
| 13 | Monitor position is directly in front of employee so employee does not have to twist head or neck |  |  |
| 14 | No glare (e.g. from windows, lights) is present on the screen which might cause employee to assume an awkward posture to read screen |  |  |

|  | Work Area The work area is designed or arranged for doing VDU tasks so that… |  |  |
| --- | --- | --- | --- |
| 15 | Thighs have clearance space between chair and VDU table/keyboard platform (thighs not trapped) |  |  |
| 16 | Legs and feet have clearance space under workstation so employee is able to get close enough to keyboard/input device |  |  |

|  | Accessories | YES | NO |
| --- | --- | --- | --- |
| 17 | Document holder, if provided, is stable and large enough to hold documents that are used |  |  |
| 18 | Document holder, if provided, is placed at about the same height and distance as monitor screen so there is little head movement when employee looks from document to screen |  |  |
| 19 | Wrist rest, if provided, is padded and free of sharp and square edges |  |  |
| 20 | Wrist rest, if provided, allows employee to keep forearms, wrists and hands straight and parallel to ground when using keyboard/input device. |  |  |
| 21 | Telephone can be used with head upright (not bent) and shoulders relaxed (not elevated) if employee does VDU tasks at the same time (i.e. using headset) |  |  |

|  | General | YES | NO |
| --- | --- | --- | --- |
| 22 | Workstation and equipment have sufficient adjustability so that the employee is able to be in a safe working posture and to make occasional changes in posture while performing VDU tasks |  |  |
| 23 | VDU Workstation, equipment and accessories are maintained in serviceable condition and function properly |  |  |

|  | Comments |  |  |
| --- | --- | --- | --- |
| Passing Score = “YES” answer on all “working postures” items (A–J) and no more than two “NO” answers on remainder of checklist (1–23) |  |  |  |